TEMP FLOOR INSTALLATION INSTRUCTIONS
TEMP FLOOR will arrive in Panels of 12 Tiles measuring 3’ X 4’ [12 square foot].


The most efficient installation methodology will involve a 2- Man Team, with one man laying out the panels in the desired Installation configuration and the other connecting one panel to the next.
CAUTION:
Always wear work gloves when handling Temp Floor panels. The plastic supports on the underside can be sharp and cause scratches or cuts to bare hands.
Connecting Temp Floor Panels
· On smooth concrete or asphalt surfaces a rubber mallet may be preferable. 

· However, on grass, sand or any non-solid surface, a steel hammer is recommended. On such surfaces some kind of support such as a wooden board underneath the joint line may be required to provide sufficient support to ensure proper connection.

· To connect one panel to the next, first ensure that the “Claw Hinges” of one panel are evenly and accurately lined-up over the plastic bars of the adjoining panel. Then apply a sharp and concise blow with the Hammer or Mallet.

NOTE:
If the claw hinge does not snap over the bar on the first try, it means it was not properly lined up. Reposition the claw over the bar ensuring a proper alignment, and try again. It will be obvious when a proper attachment has been achieved.

Disassembling Temp Floor Panels
· Temp Floor can be disassembled in the same 3’ X 4’ panels by raising the floor slightly with one hand at the joint line and sharply striking down on the adjoining tile with the other. Once disconnection has been initiated, it will continue effortlessly to the outer edge of the panel through its own momentum.

Alternatively, thanks to its’ hinge design feature, Temp Floor can be rolled up in however high or wide rolls as can be efficiently handled.
